MIPS (Microprocessor without Interlocked Pipeline Stages) is a RISC (Reduced Instruction Set Computer) architecture.

| $0 | Zero hard wired |
| --- | --- |
| $at | Assembler temporary(1) |
| $v0-$v1 | Return values (2-3) |
| $a0-$a3 | Function arguments(4-7) |
| $t0-$t7 | Temp regs ( not preserved across calls)(8-15) |
| $s0-$s7 | Saved regs(preserved across calls)(16-23) |
| $t8 $t9 | Temp regs(24-25) |
| $k0 $k1 | Reserved for kernel(26-27) |
| $gp | Global pointer(28) |
| $sp | Stack pointer(29) |
| $fp | Frame pointer (30) |
| $ra | Return address(31) |

It uses a register-based design, meaning most operations are done using registers instead of memory. Why Registers –they are on chip and thus fast than memory. Registers: 32 X 32 registers , total size of register file = 1024bits

Design Principle 1: Simplicity favors regularity.

- No of operands are always three - two to sum and third to put result in.

Keeps hardware simple, h/w for a variable no. of operands is more complicated than a fixed no. of operands.

Design Principle 2 : Smaller is faster

Design Principle 3 : Make the common case fast

- Immediate operands for common small constants , saves memory load or store operations

- $0 register for 0

Design Principle 4 : Good design demands good compromises.

MIPS uses “Big Endian” convention - Most-significant byte at least address of a word

Ranges of types of integers :

- Signed 8 bit: -128 to 127

- Unsigned 8 bit: 0 to 255

- Signed 16 bit: -32,768 to 32,767

- Unsigned 16 bit: 0 to 65,535

- Signed 32 bit: -2,147,483,648 to 2,147,483,647

- Unsigned 32 bit: 0 to 4,294,967,295

- Signed 64 bit: -9,223,372,036,854,775,808 to 9,223,372,036,854,775,807

- Unsigned 64 bit: 0 to 18,446,744,073,709,551,615

### Negative number – 2’s compliment signed int

- Some special numbers

  - 0:	0000 0000 … 0000

  - –1:	1111 1111 … 1111

- Most-negative:	1000 0000 … 0000  (−2n – 1)

- Most-positive:	0111 1111 … 1111  (+2n – 1 − 1)

- To negate – compliment all digits and add 1.

Sign Extension – replicate sign bit to left.

## MIPS Instruction Set

R Format Instructions:

Instruction fields

op: operation code (opcode)

rs: first source register number

rt: second source register number

rd: destination register number

shamt: shift amount (not used, 00000 for now)

funct: function code (extends opcode)

For eg – MIPS code – add $t0 ,$s0 ,$s1 ( $t0 = $s0+$s1)

Opcode for these is 00000

### Immediate type Instruction

### J Type Instructions

Opcode 6 bits, target 26 bits

| Type | Instruction | Mnemonic | Opcode / Funct (decimal) | Description |
| --- | --- | --- | --- | --- |
| R-type (opcode = 0) |  |  |  |  |
| R | Shift Left Logical | sll | funct = 0 | rd = rt << shamt |
| R | Shift Right Logical | srl | funct = 2 | rd = rt >> shamt (logical) |
| R | Shift Right Arithmetic | sra | funct = 3 | rd = rt >> shamt (arith.) |
| R | Shift Left Logical Variable | sllv | funct = 4 | rd = rt << (rs) |
| R | Shift Right Logical Variable | srlv | funct = 6 | rd = rt >> (rs) (logical) |
| R | Shift Right Arithmetic Variable | srav | funct = 7 | rd = rt >> (rs) (arith.) |
| R | Jump Register | jr | funct = 8 | PC = rs |
| R | Jump and Link Register | jalr | funct = 9 | rd = return addr, PC = rs |
| R | Syscall | syscall | funct = 12 | Generate system call |
| R | Breakpoint | break | funct = 13 | Breakpoint trap |
| R | Move From HI | mfhi | funct = 16 | rd = Hi |
| R | Move To HI | mthi | funct = 17 | Hi = rs |
| R | Move From LO | mflo | funct = 18 | rd = Lo |
| R | Move To LO | mtlo | funct = 19 | Lo = rs |
| R | Multiply (signed) | mult | funct = 24 | (Hi, Lo) = rs × rt |
| R | Multiply (unsigned) | multu | funct = 25 | (Hi, Lo) = rs × rt (unsigned) |
| R | Divide (signed) | div | funct = 26 | Hi = rs % rt, Lo = rs / rt |
| R | Divide (unsigned) | divu | funct = 27 | Hi = rs % rt, Lo = rs / rt (unsigned) |
| R | Add (signed) | add | funct = 32 | rd = rs + rt |
| R | Add Unsigned | addu | funct = 33 | rd = rs + rt (no overflow) |
| R | Subtract (signed) | sub | funct = 34 | rd = rs - rt |
| R | Subtract Unsigned | subu | funct = 35 | rd = rs - rt (no overflow) |
| R | AND | and | funct = 36 | rd = rs & rt |
| R | OR | or | funct = 37 | rd = rs ¦ rt |
| R | XOR | xor | funct = 38 | rd = rs ^ rt |
| R | NOR | nor | funct = 39 | rd = ~(rs ¦ rt) |
| R | Set on Less Than | slt | funct = 42 | rd = (rs < rt) ? 1 : 0 |
| R | Set on Less Than Unsigned | sltu | funct = 43 | rd = (rs < rt) ? 1 : 0 (unsigned) |
| R | No Operation | nop | funct = 0 | Same as sll $0, $0, 0 |
| I-type |  |  |  |  |
| I | Add Immediate | addi | opcode = 8 | rt = rs + imm (checks overflow) |
| I | Add Immediate Unsigned | addiu | opcode = 9 | rt = rs + imm (no overflow) |
| I | Set Less Than Immediate | slti | opcode = 10 | rt = (rs < imm) ? 1 : 0 |
| I | Set Less Than Immediate Unsigned | sltiu | opcode = 11 | rt = (rs < imm) ? 1 : 0 (unsigned) |
| I | AND Immediate | andi | opcode = 12 | rt = rs & imm (zero-extended) |
| I | OR Immediate | ori | opcode = 13 | rt = rs ¦ imm (zero-extended) |
| I | XOR Immediate | xori | opcode = 14 | rt = rs ^ imm (zero-extended) |
| I | Load Upper Immediate | lui | opcode = 15 | rt = imm << 16 |
| I | Load Byte | lb | opcode = 32 | rt = sign-extend(mem[rs + offset]) |
| I | Load Halfword | lh | opcode = 33 | rt = sign-extend(mem[rs + offset]) |
| I | Load Word | lw | opcode = 35 | rt = mem[rs + offset] |
| I | Load Byte Unsigned | lbu | opcode = 36 | rt = zero-extend(mem[rs + offset]) |
| I | Load Halfword Unsigned | lhu | opcode = 37 | rt = zero-extend(mem[rs + offset]) |
| I | Store Byte | sb | opcode = 40 | mem[rs + offset] = rt (byte) |
| I | Store Halfword | sh | opcode = 41 | mem[rs + offset] = rt (halfword) |
| I | Store Word | sw | opcode = 43 | mem[rs + offset] = rt (word) |
| I | Branch on Equal | beq | opcode = 4 | if (rs == rt) PC += offset |
| I | Branch on Not Equal | bne | opcode = 5 | if (rs != rt) PC += offset |
| I | Branch on Less Than Zero | bltz | opcode = 1 | if (rs < 0) PC += offset |
| I | Branch on Greater or Equal Zero | bgez | opcode = 1 | if (rs ≥ 0) PC += offset (rt=1) |
| I | Load Word Coprocessor 0 | lwc0 | opcode = 48 | Load word to CP0 register |
| I | Load Word Coprocessor 1 | lwc1 | opcode = 49 | Load word to CP1 (FPU) register |
| I | Store Word Coprocessor 0 | swc0 | opcode = 56 | Store word from CP0 register |
| I | Store Word Coprocessor 1 | swc1 | opcode = 57 | Store word from CP1 (FPU) register |
| J-type | Target address 26 bits |  | Opcode 6 bits |  |
| J | Jump | j | opcode = 2 | Jump to target address |
| J | Jump and Link | jal | opcode = 3 | $ra = PC + 4, then jump to target address |

### Procedure Calling

Steps happening:

- Place parameters (x, y, z) in registers

- Transfer control to procedure (or method)

- Acquire storage for procedure

- Perform procedure’s operations

- Place result in register for caller

- Return to place of call

Memory Hierarchy

Principle of locality – data once used might be used again

Spatial Locality- data once used, then its nearby items may be used in future, hence a block is transferred to cache as cache line

Temporal Locality – data once used might be used again in future, so copied to cache.

- Static RAM (SRAM, used by cache)

  - 0.5ns – 2.5ns, $2000 – $5000 per GB

  - Storage Mechanism: Each bit of data is stored using a flip-flop (a bistable latching circuit), which typically consists of four to six transistors (4T or 6T).

  - Data Retention: It is called Static because, as long as power is supplied, the flip-flop maintains its state without needing to be rewritten. Hence fast because there is no refresh delays as in DRAM

- Dynamic RAM (DRAM)      ≈ 100 times

  - 50ns – 70ns, $20 – $75 per GB

  - Storage Mechanism: Each bit of data is stored using a simple memory cell consisting of one transistor and one capacitor (1T1C).

  - Data Retention: It is called Dynamic because the capacitor's charge, which holds the data (1 or 0), leaks away over time. Therefore, the data must be periodically refreshed (read and then rewritten) every few milliseconds by a memory controller(refresh cycles interrupt  data access and makes it slow)

  - high capacity and low cost, but it requires a constant refresh process.

- Magnetic disk                     ≈ 100 times

  - 5ms – 20ms, $0.20 – $2 per GB

- Ideal memory:

  - Access time of cache

  - Capacity and cost/GB of disk

Disk average read time

Average Read Time=Seek Time+Rotational Latency+Transfer Time+Controller Overhead

Average rotational latency is half  the time for one full rotation.

Transfer Time=Data SizeTransfer Rate=512 B100,000,000 B/s≈0.00000512 s

Direct mapped cache – determine cache address : The address is split from right to left into three field , - tag, index, block offset